- Install Unreal Engine V4.27.2 from Epic Games Launcher(has to be installed first).

- Install Visual Studio Community 2022:

- An exe file from

https://visualstudio.microsoft.com/thank-you-downloading-visual-studio/?sku=Community&channel=Release&version=VS2022&source=VSLandingPage&cid=2030&passive=false

- In Visual Studio installer, besides Visual Studio community 2022 -> Click on import configuration and select the .vsconfig file. Then click on install.

- Install git:

- Navigate to the folder where you want to install AirSim -> open cmd there -> git clone https://github.com/microsoft/AirSim

OR

Use GitHub Desktop to clone the repository.

- Now open Developer Command Prompt for VS 2022(run as administrator).

- cd /d <AirSim directory path>

- build.cmd

Now let everything install.

- Go to Epic games marketplace(Now FAB) and add an environment to use for simulation. Click on add to library.

- Go to Epic Games Launcher -> Unreal Engine -> Library -> Fab Library

Click on Create Project and let it download.

Then create a project with that environment.

- File -> New C++ Class -> none

- A .sln VS file will be created in the project folder.

- Save and close the UE project.

- Go to AirSim directory. Copy Unreal folder and paste it in project folder.

- Change .uproject file as given in the Final Sim folder.

- Similarly change .ini file

- Right click on .uproject file -> Generate Visual Studio project files.

- To run the UE simulation project, ALWAYS RUN IT FROM VISUAL STUDIO.

- In Unreal Engine, add player start object wherever you want.

AT THIS POINT, THE SIMULATION WILL RUN WHEN YOU CLICK ON PLAY. AFTER EDITING .JSON FILES AND SETTING UP ARDUPILOT, THE SIMULATION WILL NOT PLAY UNTIL AND UNLESS SUCCESSFULLY CONNECTED TO ARDUILOT.

- Now install  wsl and ardupilot using instructions from Ardupilot WSL.txt file.

MAKE SURE TO USE UBUNTU 20.04

- Copy the settings.json file to AirSim folder created automatically in documents.

- In .json file change